#include <Stepper.h>

#include <WiFi.h>
#include <WebServer.h>

// WiFi AP credentials
const char* ssid = "ESP32-AP";
const char* password = "12345678";
#include "DFRobotDFPlayerMini.h"
//#include <HardwareSerial.h>
//HardwareSerial Serial2(2); // RX:gpio16 TX:gpio17
int obezd = 0;
int t = 24;
#define HC_TRIG 4
#define HC_ECHO 2

Stepper stepper(510, 32, 0, 33, 15);

DFRobotDFPlayerMini myDFPlayer;
// Create web server on port 80
WebServer server(80);

// L298N motor driver pins
const int ENA = 26; // Left motor PWM (speed)
const int IN1 = 27; // Left motor direction 1
const int IN2 = 25; // Left motor direction 2
const int ENB = 14; // Right motor PWM (speed)
const int IN3 = 12; // Right motor direction 1
const int IN4 = 13; // Right motor direction 2

// Variables to store command and speed
String currentCommand = "";
int currentSpeed = 0;
const int defaultSpeed = 100; // Default speed when no value is provided

int getMm(uint8_t trig, uint8_t echo, int t) {
// импульс 10 мкс
digitalWrite(trig, HIGH);
delayMicroseconds(10);
digitalWrite(trig, LOW);

// измеряем время ответного импульса
uint32_t us = pulseIn(echo, HIGH);

// считаем расстояние и возвращаем
return us * (t * 6 / 10 + 330) / 2000ul;
}

void setup() {
// Start Serial
Serial.begin(115200);
delay(1000);
Serial2.begin(9600, SERIAL_8N1, 16, 17);
myDFPlayer.begin(Serial2);
myDFPlayer.volume(25);
pinMode(HC_TRIG, OUTPUT); // trig выход
pinMode(HC_ECHO, INPUT); // echo вход
stepper.setSpeed(60);
// Set up motor pins
pinMode(ENA, OUTPUT);
pinMode(IN1, OUTPUT);
pinMode(IN2, OUTPUT);
pinMode(ENB, OUTPUT);
pinMode(IN3, OUTPUT);
pinMode(IN4, OUTPUT);
myDFPlayer.play(1);
// Set up Access Point
WiFi.softAP(ssid, password);

Serial.println();
Serial.print("AP IP address: ");
Serial.println(WiFi.softAPIP());

// Define server routes
server.onNotFound(handleCommand);

// Start server
server.begin();
Serial.println("HTTP server started");
}

void loop() {
server.handleClient();
int dist = getMm(HC_TRIG, HC_ECHO, t); // получаем расстояние в мм
delay(50);
if (dist < 300 and obezd == 1){
digitalWrite(IN1, LOW); // Left motor backward
digitalWrite(IN2, LOW);
digitalWrite(IN3, LOW); // Right motor backward
digitalWrite(IN4, LOW);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
stepper.step(255);
int dist1 = getMm(HC_TRIG, HC_ECHO, t); // получаем расстояние в мм
delay(50);
stepper.step(-510);
int dist2 = getMm(HC_TRIG, HC_ECHO, t); // получаем расстояние в мм
delay(50);
stepper.step(255);
digitalWrite(32, LOW);
digitalWrite(0, LOW);
digitalWrite(33, LOW);
digitalWrite(15, LOW);
if (dist1 < dist2){
digitalWrite(IN1, LOW); // Left motor backward
digitalWrite(IN2, HIGH);
digitalWrite(IN3, LOW); // Right motor backward
digitalWrite(IN4, HIGH);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
delay(1000);
digitalWrite(IN1, HIGH); // Left motor forward
digitalWrite(IN2, LOW);
digitalWrite(IN3, LOW); // Right motor backward
digitalWrite(IN4, HIGH);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
delay(1500);
digitalWrite(IN1, HIGH); // Left motor forward
digitalWrite(IN2, LOW);
digitalWrite(IN3, HIGH); // Right motor forward
digitalWrite(IN4, LOW);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
}
else {
digitalWrite(IN1, LOW); // Left motor backward
digitalWrite(IN2, HIGH);
digitalWrite(IN3, LOW); // Right motor backward
digitalWrite(IN4, HIGH);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
delay(1000);
digitalWrite(IN1, LOW); // Left motor forward
digitalWrite(IN2, HIGH);
digitalWrite(IN3, HIGH); // Right motor backward
digitalWrite(IN4, LOW);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);

delay(1500);
digitalWrite(IN1, HIGH); // Left motor forward
digitalWrite(IN2, LOW);
digitalWrite(IN3, HIGH); // Right motor forward
digitalWrite(IN4, LOW);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
}
}
}

void handleCommand() {
String request = server.uri();

if (request.startsWith("/")) {
request = request.substring(1);
}

// Parse command and speed (if provided)
if (request.indexOf(":") != -1) {
int colonIndex = request.indexOf(":");
currentCommand = request.substring(0, colonIndex);
String speedStr = request.substring(colonIndex + 1);
currentSpeed = speedStr.toInt();
} else {
currentCommand = request;
currentSpeed = (currentCommand == "S") ? 0 : defaultSpeed; // Default speed unless "S"
}

// Process command and control robot
controlRobot();
displayCommand();

// Send response back to client
server.send(200, "text/plain", "Command received: " + request);
}

void controlRobot() {
// Stop motors by default
analogWrite(ENA, 0);
analogWrite(ENB, 0);
digitalWrite(IN1, LOW);
digitalWrite(IN2, LOW);
digitalWrite(IN3, LOW);
digitalWrite(IN4, LOW);

if (currentCommand == "F") { // Forward
digitalWrite(IN1, HIGH); // Left motor forward
digitalWrite(IN2, LOW);
digitalWrite(IN3, HIGH); // Right motor forward
digitalWrite(IN4, LOW);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
} 
else if (currentCommand == "B") { // Backward
digitalWrite(IN1, LOW); // Left motor backward
digitalWrite(IN2, HIGH);
digitalWrite(IN3, LOW); // Right motor backward
digitalWrite(IN4, HIGH);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
} 
else if (currentCommand == "L") { // Left
digitalWrite(IN1, LOW); // Left motor backward
digitalWrite(IN2, HIGH);
digitalWrite(IN3, HIGH); // Right motor forward
digitalWrite(IN4, LOW);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
} 
else if (currentCommand == "next") {
myDFPlayer.next();
delay(1000);
}
else if (currentCommand == "last") {
myDFPlayer.previous();
delay(1000);
}
else if (currentCommand == "+") {
myDFPlayer.volumeUp();
delay(1000);
}
else if (currentCommand == "-") {
myDFPlayer.volumeDown();
delay(1000);
}
else if (currentCommand == "h+") {
obezd = 1;
}
else if (currentCommand == "h-") {
obezd = 0;
}
else if (currentCommand == "R") { // Right
digitalWrite(IN1, HIGH); // Left motor forward
digitalWrite(IN2, LOW);
digitalWrite(IN3, LOW); // Right motor backward
digitalWrite(IN4, HIGH);
analogWrite(ENA, currentSpeed);
analogWrite(ENB, currentSpeed);
} 
else if (currentCommand == "S") { // Stop
analogWrite(ENA, 0);
analogWrite(ENB, 0);
}
}

void displayCommand() {
Serial.println("-------------------");
Serial.print("Received Command: ");
Serial.println(currentCommand);

if (currentSpeed > 0) {
Serial.print("Speed Value: ");
Serial.println(currentSpeed);
}

if (currentCommand == "F") {
Serial.println("Action: Moving Forward");
} else if (currentCommand == "B") {
Serial.println("Action: Moving Backward");
} else if (currentCommand == "L") {
Serial.println("Action: Turning Left");
} else if (currentCommand == "R") {
Serial.println("Action: Turning Right");
} else if (currentCommand == "S") {
Serial.println("Action: Stop");
}
Serial.println("-------------------");